ClawLink 三级协同智能硬件系统全面解析

# ClawLink-TriLayer 三级协同智能硬件系统

## 项目概述

基于「OpenClaw大脑层 + 树莓派脊椎层 + ESP32末梢层」三级架构打造的全开源智能硬件控制系统，实现复杂任务调度→边缘智能处理→端侧硬件执行 的全链路闭环，支持离线自治、多设备协同、模块化扩展。

### 核心特性

🌐 跨层通信：MQTT/ESP-NOW/WebSocket 多协议适配

🧠 分层智能：大脑层复杂决策、脊椎层边缘推理、末梢层实时执行

📱 全离线：断网状态下树莓派+ESP32可完成核心功能闭环

🛠️ 易扩展：新增传感器/执行器仅需适配末梢层，无需修改核心架构

📝 开源协议：MIT License（完全自由修改/商用）

## 完整项目目录结构

Plain Text ClawLink-TriLayer/ ├── LICENSE # MIT开源协议 ├── README.md # 项目说明 ├── docker-compose.yml # 树莓派服务一键部署 ├── .env.example # 环境变量模板 ├── docs/ # 文档目录 │ ├── 快速上手.md │ ├── 硬件接线指南.md │ └── 二次开发指南.md ├── openclaw-brain/ # OpenClaw大脑层 │ ├── clawlink_trilayer.skill.md # 核心调度插件 │ └── install_skill.sh # 插件安装脚本 ├── raspberrypi-spine/ # 树莓派脊椎层 │ ├── backend/ # 后端服务 │ │ ├── Dockerfile │ │ ├── requirements.txt │ │ └── src/ │ │ ├── main.py # FastAPI入口 │ │ ├── mqtt_client.py # MQTT边缘网关 │ │ ├── llm_agent.py # 本地LLM推理（Ollama） │ │ ├── vision_engine.py # YOLOv8视觉处理 │ │ └── device_manager.py # 设备管理 │ └── frontend/ # 极简Web前端（Vue3） │ ├── index.html │ ├── app.js │ └── style.css ├── esp32-periphery/ # ESP32末梢层 │ ├── main/ │ │ ├── CMakeLists.txt │ │ ├── sdkconfig.defaults │ │ ├── main.c # 固件入口 │ │ ├── mqtt_comm.c # MQTT通信 │ │ ├── sensors.c # 传感器采集 │ │ └── actuators.c # 执行器控制 │ └── flash.sh # 固件烧录脚本 └── examples/ # 示例代码 ├── task_demo.py # 复杂任务调度示例 └── sensor_extend.c # 新增传感器示例

## 一、OpenClaw 大脑层代码

### 1. 核心调度插件 clawlink_trilayer.skill.md

Markdown # ClawLink-TriLayer 三级协同调度插件 ## 插件信息 - 名称: clawlink_trilayer - 版本: 1.0.0 - 描述: 对接树莓派脊椎层，实现复杂任务拆解与跨设备调度 - 依赖: mqtt, requests ## 配置项 ```toml [clawlink_trilayer] mqtt_broker = "你的树莓派IP" mqtt_port = 1883 mqtt_user = "clawlink-upper" mqtt_pass = "ClawLinkUpper@2026" raspberrypi_api = "http://你的树莓派IP:8001"

## 指令映射

| 自然语言指令 | 处理逻辑 |
| --- | --- |
| "采集所有设备温湿度并汇总" | 调用树莓派API获取所有ESP32传感器数据，汇总后返回 |
| "控制设备claw_01打开机械爪" | 通过MQTT下发指令到树莓派，转发至指定ESP32 |
| "检测前方障碍物，有障碍则停止移动底盘" | 触发树莓派视觉引擎，联动ESP32执行器控制 |

## 核心代码

Python import paho.mqtt.client as mqtt import requests import json # MQTT客户端初始化 def init_mqtt(config): client = mqtt.Client() client.username_pw_set(config['mqtt_user'], config['mqtt_pass']) client.connect(config['mqtt_broker'], config['mqtt_port'], 60) client.loop_start() return client # 任务拆解与下发 def handle_task(command, config): client = init_mqtt(config) api_url = config['raspberrypi_api'] # 1. 解析指令 if "采集温湿度" in command: # 调用树莓派API获取传感器数据 resp = requests.get(f"{api_url}/api/devices/sensors") if resp.status_code == 200: data = resp.json() return f"温湿度数据汇总：{json.dumps(data, ensure_ascii=False)}" else: return "获取传感器数据失败" elif "打开机械爪" in command: # 提取设备ID device_id = "claw_01" # 实际场景可通过NLP提取 # 下发指令到树莓派 client.publish( f"clawlink/spine/command", json.dumps({ "device_id": device_id, "action": "servo_control", "params": {"angle": 90, "pin": 18} }) ) return f"已下发指令：打开设备{device_id}机械爪" elif "检测障碍物" in command: # 触发视觉检测 resp = requests.post(f"{api_url}/api/vision/detect", json={"target": "obstacle"}) if resp.status_code == 200: result = resp.json() if result["has_obstacle"]: # 下发停止指令 client.publish( f"clawlink/spine/command", json.dumps({ "device_id": "chassis_01", "action": "motor_stop" }) ) return "检测到障碍物，已停止移动底盘" else: return "未检测到障碍物，底盘正常运行" else: return "不支持的指令，请重试" # OpenClaw插件入口 def skill_handler(command, context): config = context['config']['clawlink_trilayer'] return handle_task(command, config)

### 2. 插件安装脚本 install_skill.sh

Bash #!/bin/bash # OpenClaw插件安装脚本 # 创建插件目录 mkdir -p ~/.openclaw/skills # 复制插件文件 cp clawlink_trilayer.skill.md ~/.openclaw/skills/ # 重启OpenClaw服务 openclaw restart echo "ClawLink-TriLayer插件安装完成！" echo "请修改 ~/.openclaw/config.toml 配置mqtt_broker等参数"

## 二、树莓派 脊椎层代码

### 1. 后端依赖 requirements.txt

Plain Text fastapi==0.104.1 uvicorn==0.24.0 paho-mqtt==1.6.1 ollama==0.1.9 ultralytics==8.0.224 python-dotenv==1.0.0 pandas==2.1.4

### 2. MQTT客户端 mqtt_client.py

Python import paho.mqtt.client as mqtt import json import os from dotenv import load_dotenv load_dotenv() # MQTT配置 MQTT_BROKER = os.getenv("MQTT_BROKER", "localhost") MQTT_PORT = int(os.getenv("MQTT_PORT", 1883)) MQTT_USER = os.getenv("MQTT_EDGE_USER", "clawlink-edge") MQTT_PASS = os.getenv("MQTT_EDGE_PASS", "ClawLinkEdge@2026") # 设备状态缓存 device_states = {} # MQTT回调函数 def on_connect(client, userdata, flags, rc): print(f"MQTT连接成功，状态码：{rc}") # 订阅末梢层上报主题 client.subscribe("clawlink/periphery/#") # 订阅大脑层指令主题 client.subscribe("clawlink/spine/command") def on_message(client, userdata, msg): topic = msg.topic payload = json.loads(msg.payload.decode()) print(f"收到消息：{topic} -> {payload}") if topic.startswith("clawlink/periphery/report"): # 处理末梢层状态上报 device_id = topic.split("/")[2] device_states[device_id] = payload elif topic == "clawlink/spine/command": # 转发指令到指定末梢设备 device_id = payload["device_id"] client.publish( f"clawlink/periphery/command/{device_id}", json.dumps(payload) ) # 初始化MQTT客户端 def init_mqtt_client(): client = mqtt.Client() client.username_pw_set(MQTT_USER, MQTT_PASS) client.on_connect = on_connect client.on_message = on_message client.connect(MQTT_BROKER, MQTT_PORT, 60) client.loop_start() return client # 发布指令到末梢层 def publish_to_periphery(device_id, action, params): client = init_mqtt_client() client.publish( f"clawlink/periphery/command/{device_id}", json.dumps({ "action": action, "params": params }) )

### 3. 本地LLM推理 llm_agent.py

Python import ollama import os from dotenv import load_dotenv load_dotenv() OLLAMA_URL = os.getenv("OLLAMA_BASE_URL", "http://localhost:11434/api") DEFAULT_MODEL = os.getenv("OLLAMA_DEFAULT_MODEL", "llama3:8b-instruct-q4_0") # 初始化Ollama客户端 ollama_client = ollama.Client(host=OLLAMA_URL) # 本地LLM推理（边缘决策） def llm_infer(prompt): try: response = ollama_client.chat( model=DEFAULT_MODEL, messages=[{"role": "user", "content": prompt}] ) return response["message"]["content"] except Exception as e: return f"LLM推理失败：{str(e)}" # 任务优先级决策 def task_priority_decision(tasks): prompt = f""" 请对以下硬件控制任务进行优先级排序（1-最高，数字越小优先级越高），并说明理由： {json.dumps(tasks, ensure_ascii=False)} 输出格式：JSON，包含tasks（排序后的任务列表，带priority字段）和reason字段 """ return llm_infer(prompt)

### 4. 视觉处理引擎 vision_engine.py

Python from ultralytics import YOLO import cv2 import os from dotenv import load_dotenv load_dotenv() # 加载YOLOv8模型 model = YOLO("yolov8n.pt") CAMERA_INDEX = int(os.getenv("CAMERA_INDEX", 0)) # 障碍物检测 def detect_obstacle(): cap = cv2.VideoCapture(CAMERA_INDEX) ret, frame = cap.read() if not ret: return {"has_obstacle": False, "error": "摄像头读取失败"} # 推理（检测人、椅子、箱子等障碍物） results = model(frame) obstacle_classes = [0, 5, 27] # COCO类别：人、椅子、箱子 detections = results[0].boxes.cls.cpu().numpy() has_obstacle = any(cls in obstacle_classes for cls in detections) cap.release() return { "has_obstacle": has_obstacle, "detections": len(detections), "obstacle_classes": obstacle_classes } # 视觉抓取目标定位 def detect_target(target_name): cap = cv2.VideoCapture(CAMERA_INDEX) ret, frame = cap.read() if not ret: return {"success": False, "error": "摄像头读取失败"} results = model(frame) # 查找目标类别 class_names = model.names target_cls = [k for k, v in class_names.items() if v == target_name] if not target_cls: return {"success": False, "error": "目标类别不存在"} # 获取目标坐标 boxes = results[0].boxes target_boxes = [box for box in boxes if box.cls.cpu().numpy()[0] == target_cls[0]] cap.release() if not target_boxes: return {"success": False, "message": "未检测到目标"} # 返回第一个目标的中心坐标 box = target_boxes[0].xyxy.cpu().numpy()[0] center_x = (box[0] + box[2]) / 2 center_y = (box[1] + box[3]) / 2 return { "success": True, "center_x": center_x, "center_y": center_y, "box": box.tolist() }

### 5. FastAPI入口 main.py

Python from fastapi import FastAPI, HTTPException from fastapi.middleware.cors import CORSMiddleware import uvicorn import json from mqtt_client import init_mqtt_client, device_states, publish_to_periphery from llm_agent import llm_infer, task_priority_decision from vision_engine import detect_obstacle, detect_target # 初始化FastAPI app = FastAPI(title="ClawLink-TriLayer 脊椎层API") app.add_middleware( CORSMiddleware, allow_origins=["*"], allow_credentials=True, allow_methods=["*"], allow_headers=["*"], ) # 初始化MQTT mqtt_client = init_mqtt_client() # 设备管理接口 @app.get("/api/devices") def get_devices(): """获取所有在线设备""" return {"devices": list(device_states.keys()), "count": len(device_states)} @app.get("/api/devices/sensors") def get_sensors(): """获取所有设备传感器数据""" sensor_data = {} for dev_id, state in device_states.items(): if "sensors" in state: sensor_data[dev_id] = state["sensors"] return sensor_data # 视觉处理接口 @app.post("/api/vision/detect") def vision_detect(data: dict): """视觉检测接口""" target = data.get("target", "obstacle") if target == "obstacle": return detect_obstacle() else: return detect_target(target) # LLM推理接口 @app.post("/api/llm/infer") def llm_infer_api(data: dict): """本地LLM推理接口""" prompt = data.get("prompt") if not prompt: raise HTTPException(status_code=400, detail="prompt不能为空") return {"result": llm_infer(prompt)} # 设备控制接口 @app.post("/api/devices/control") def control_device(data: dict): """控制末梢设备""" device_id = data.get("device_id") action = data.get("action") params = data.get("params", {}) if not device_id or not action: raise HTTPException(status_code=400, detail="device_id和action不能为空") publish_to_periphery(device_id, action, params) return {"success": True, "message": f"已下发{action}指令到设备{device_id}"} # 启动服务 if __name__ == "__main__": uvicorn.run( "main:app", host="0.0.0.0", port=8001, reload=True )

### 6. 极简Web前端 index.html

HTML <!DOCTYPE html> <html lang="zh-CN"> <head> <meta charset="UTF-8"> <meta name="viewport" content="width=device-width, initial-scale=1.0"> <title>ClawLink-TriLayer 控制中心</title> <link rel="stylesheet" href="style.css"> </head> <body> <div class="container"> <h1>ClawLink-TriLayer 控制中心</h1> <!-- 设备列表 --> <div class="device-section"> <h2>在线设备</h2> <div id="device-list"></div> </div> <!-- 传感器数据 --> <div class="sensor-section"> <h2>传感器数据</h2> <div id="sensor-data"></div> </div> <!-- 设备控制 --> <div class="control-section"> <h2>设备控制</h2> <select id="device-select"> <option value="">选择设备</option> </select> <select id="action-select"> <option value="servo_control">舵机控制</option> <option value="motor_control">电机控制</option> <option value="sensor_read">读取传感器</option> </select> <input type="text" id="params-input" placeholder="参数（JSON格式）" value='{"angle": 90, "pin": 18}'> <button onclick="controlDevice()">执行</button> </div> <!-- 视觉检测 --> <div class="vision-section"> <h2>视觉检测</h2> <button onclick="detectObstacle()">检测障碍物</button> <div id="vision-result"></div> </div> </div> <script src="app.js"></script> </body> </html>

### 7. 前端逻辑 app.js

JavaScript // API基础地址 const API_BASE = "http://" + window.location.hostname + ":8001/api"; // 初始化页面 window.onload = async () => { await loadDevices(); await loadSensors(); }; // 加载设备列表 async function loadDevices() { const resp = await fetch(`${API_BASE}/devices`); const data = await resp.json(); const deviceList = document.getElementById("device-list"); const deviceSelect = document.getElementById("device-select"); deviceList.innerHTML = JSON.stringify(data, null, 2); data.devices.forEach(dev => { const option = document.createElement("option"); option.value = dev; option.textContent = dev; deviceSelect.appendChild(option); }); } // 加载传感器数据 async function loadSensors() { const resp = await fetch(`${API_BASE}/devices/sensors`); const data = await resp.json(); document.getElementById("sensor-data").innerHTML = JSON.stringify(data, null, 2); } // 控制设备 async function controlDevice() { const deviceId = document.getElementById("device-select").value; const action = document.getElementById("action-select").value; const params = JSON.parse(document.getElementById("params-input").value); if (!deviceId) { alert("请选择设备"); return; } const resp = await fetch(`${API_BASE}/devices/control`, { method: "POST", headers: { "Content-Type": "application/json" }, body: JSON.stringify({ device_id: deviceId, action: action, params: params }) }); const data = await resp.json(); alert(data.message); // 刷新设备状态 await loadSensors(); } // 检测障碍物 async function detectObstacle() { const resp = await fetch(`${API_BASE}/vision/detect`, { method: "POST", headers: { "Content-Type": "application/json" }, body: JSON.stringify({ target: "obstacle" }) }); const data = await resp.json(); document.getElementById("vision-result").innerHTML = JSON.stringify(data, null, 2); }

## 三、ESP32 末梢层代码

### 1. 固件入口 main.c

C #include <stdio.h> #include "freertos/FreeRTOS.h" #include "freertos/task.h" #include "mqtt_comm.h" #include "sensors.h" #include "actuators.h" #define DEVICE_ID "claw_01" // 设备唯一ID #define WIFI_SSID "你的WiFi名称" #define WIFI_PASS "你的WiFi密码" #define MQTT_BROKER "你的树莓派IP" #define MQTT_PORT 1883 void app_main(void) { // 1. 初始化WiFi wifi_init(WIFI_SSID, WIFI_PASS); // 2. 初始化MQTT mqtt_client_init(MQTT_BROKER, MQTT_PORT, DEVICE_ID); // 3. 初始化传感器 sensors_init(); // 4. 初始化执行器 actuators_init(); // 5. 启动传感器采集任务 xTaskCreate(sensor_collect_task, "sensor_collect", 4096, NULL, 5, NULL); // 6. 启动MQTT消息处理任务 xTaskCreate(mqtt_message_handler_task, "mqtt_handler", 4096, NULL, 5, NULL); // 7. 启动心跳上报任务 xTaskCreate(heartbeat_task, "heartbeat", 2048, NULL, 4, NULL); }

### 2. MQTT通信 mqtt_comm.c

C #include <stdio.h> #include <string.h> #include "freertos/FreeRTOS.h" #include "freertos/task.h" #include "esp_log.h" #include "mqtt_client.h" #include "wifi_provisioning/manager.h" static const char *TAG = "MQTT_COMM"; static esp_mqtt_client_handle_t mqtt_client; static char device_id[32]; // WiFi初始化 void wifi_init(const char *ssid, const char *pass) { // 简化版WiFi初始化（实际项目请使用ESP-IDF标准WiFi初始化） ESP_LOGI(TAG, "WiFi初始化中..."); vTaskDelay(2000 / portTICK_PERIOD_MS); ESP_LOGI(TAG, "WiFi连接成功"); } // MQTT连接回调 static void mqtt_event_handler(void *handler_args, esp_event_base_t base, int32_t event_id, void *event_data) { esp_mqtt_event_handle_t event = event_data; esp_mqtt_client_handle_t client = event->client; switch ((esp_mqtt_event_id_t)event_id) { case MQTT_EVENT_CONNECTED: ESP_LOGI(TAG, "MQTT连接成功"); // 订阅指令主题 char cmd_topic[64]; snprintf(cmd_topic, sizeof(cmd_topic), "clawlink/periphery/command/%s", device_id); esp_mqtt_client_subscribe(client, cmd_topic, 0); break; case MQTT_EVENT_DATA: ESP_LOGI(TAG, "收到MQTT消息：%.*s", event->data_len, event->data); // 解析指令并处理 handle_mqtt_command(event->data, event->data_len); break; case MQTT_EVENT_DISCONNECTED: ESP_LOGI(TAG, "MQTT断开连接"); break; default: break; } } // 初始化MQTT客户端 void mqtt_client_init(const char *broker, int port, const char *dev_id) { strncpy(device_id, dev_id, sizeof(device_id) - 1); esp_mqtt_client_config_t mqtt_cfg = { .broker.address.uri = broker, .broker.address.port = port, .credentials.username = "clawlink-device", .credentials.authentication.password = "ClawLinkDevice@2026", .client.id = dev_id, }; mqtt_client = esp_mqtt_client_init(&mqtt_cfg); esp_mqtt_client_register_event(mqtt_client, ESP_EVENT_ANY_ID, mqtt_event_handler, NULL); esp_mqtt_client_start(mqtt_client); } // 发布消息到MQTT void mqtt_publish(const char *topic, const char *payload) { if (mqtt_client == NULL) { ESP_LOGE(TAG, "MQTT客户端未初始化"); return; } int msg_id = esp_mqtt_client_publish(mqtt_client, topic, payload, 0, 1, 0); ESP_LOGI(TAG, "发布消息ID：%d", msg_id); } // 处理MQTT指令 void handle_mqtt_command(const char *data, int len) { // 简化版JSON解析（实际项目建议使用cJSON） char action[32] = {0}; char params[128] = {0}; // 提取action和params（示例：{"action":"servo_control","params":{"angle":90,"pin":18}}） if (strstr(data, "servo_control")) { strcpy(action, "servo_control"); // 提取角度和引脚 int angle = 90, pin = 18; sscanf(data, "%*[^0-9]%d%*[^0-9]%d", &angle, &pin); servo_control(pin, angle); } else if (strstr(data, "motor_control")) { strcpy(action, "motor_control"); motor_control(1, 100); // 启动电机，占空比100 } else if (strstr(data, "sensor_read")) { strcpy(action, "sensor_read"); // 上报传感器数据 sensor_data_t sensor_data = get_sensor_data(); char payload[256]; snprintf(payload, sizeof(payload), "{\"device_id\":\"%s\",\"sensors\":{\"temp\":%.2f,\"hum\":%.2f,\"ultrasonic\":%.2f}}", device_id, sensor_data.temp, sensor_data.hum, sensor_data.ultrasonic); char report_topic[64]; snprintf(report_topic, sizeof(report_topic), "clawlink/periphery/report/%s", device_id); mqtt_publish(report_topic, payload); } ESP_LOGI(TAG, "执行指令：%s", action); } // 心跳上报任务 void heartbeat_task(void *pvParameters) { while (1) { char payload[128]; snprintf(payload, sizeof(payload), "{\"device_id\":\"%s\",\"status\":\"online\"}", device_id); char heartbeat_topic[64]; snprintf(heartbeat_topic, sizeof(heartbeat_topic), "clawlink/periphery/heartbeat/%s", device_id); mqtt_publish(heartbeat_topic, payload); vTaskDelay(5000 / portTICK_PERIOD_MS); // 每5秒上报一次 } } // MQTT消息处理任务 void mqtt_message_handler_task(void *pvParameters) { while (1) { vTaskDelay(100 / portTICK_PERIOD_MS); // 空闲延时 } }

### 3. 传感器采集 sensors.c

C #include <stdio.h> #include "freertos/FreeRTOS.h" #include "freertos/task.h" #include "driver/i2c.h" #include "driver/gpio.h" #include "esp_log.h" static const char *TAG = "SENSORS"; // 传感器数据结构体 typedef struct { float temp; // 温度 float hum; // 湿度 float ultrasonic; // 超声波距离 } sensor_data_t; static sensor_data_t current_sensor_data = {0}; // AHT20温湿度传感器读取 static void aht20_read(float *temp, float *hum) { // 简化版AHT20读取（实际项目请使用官方驱动） *temp = 25.5 + (rand() % 100) / 10.0; // 模拟温度 *hum = 60.2 + (rand() % 50) / 10.0; // 模拟湿度 } // HC-SR04超声波传感器读取 static float hc_sr04_read(void) { // 简化版超声波读取（实际项目请使用GPIO驱动） return 50.0 + (rand() % 100) / 2.0; // 模拟距离（cm） } // 传感器初始化 void sensors_init(void) { ESP_LOGI(TAG, "传感器初始化完成"); // 实际项目中初始化I2C、GPIO等 } // 获取传感器数据 sensor_data_t get_sensor_data(void) { return current_sensor_data; } // 传感器采集任务 void sensor_collect_task(void *pvParameters) { while (1) { // 读取温湿度 aht20_read(&current_sensor_data.temp, &current_sensor_data.hum); // 读取超声波距离 current_sensor_data.ultrasonic = hc_sr04_read(); ESP_LOGI(TAG, "温度：%.2f℃，湿度：%.2f%%，距离：%.2fcm", current_sensor_data.temp, current_sensor_data.hum, current_sensor_data.ultrasonic); vTaskDelay(2000 / portTICK_PERIOD_MS); // 每2秒采集一次 } }

### 4. 执行器控制 actuators.c

C #include <stdio.h> #include "freertos/FreeRTOS.h" #include "freertos/task.h" #include "driver/ledc.h" #include "driver/gpio.h" #include "esp_log.h" static const char *TAG = "ACTUATORS"; // 舵机初始化 static void servo_init(int pin) { // 配置LEDC PWM ledc_timer_config_t timer_conf = { .speed_mode = LEDC_LOW_SPEED_MODE, .timer_num = LEDC_TIMER_0, .duty_resolution = LEDC_TIMER_12_BIT, .freq_hz = 50, // 50Hz（舵机标准频率） .clk_cfg = LEDC_AUTO_CLK, }; ledc_timer_config(&timer_conf); ledc_channel_config_t channel_conf = { .channel = LEDC_CHANNEL_0, .speed_mode = LEDC_LOW_SPEED_MODE, .timer_sel = LEDC_TIMER_0, .intr_type = LEDC_INTR_DISABLE, .gpio_num = pin, .duty = 0, .hpoint = 0, }; ledc_channel_config(&channel_conf); } // 舵机控制 void servo_control(int pin, int angle) { // 角度转PWM占空比（0-180°对应500-2500us） int duty = (angle * 2000 / 180) + 500; duty = (duty * 4096) / 20000; // 转换为12位占空比 servo_init(pin); ledc_set_duty(LEDC_LOW_SPEED_MODE, LEDC_CHANNEL_0, duty); ledc_update_duty(LEDC_LOW_SPEED_MODE, LEDC_CHANNEL_0); ESP_LOGI(TAG, "舵机控制：引脚%d，角度%d°，占空比%d", pin, angle, duty); } // 电机控制 void motor_control(int enable, int duty) { if (enable) { ESP_LOGI(TAG, "电机启动，占空比：%d%%", duty); } else { ESP_LOGI(TAG, "电机关闭"); } } // 执行器初始化 void actuators_init(void) { ESP_LOGI(TAG, "执行器初始化完成"); // 实际项目中初始化GPIO、PWM等 } // 电机停止 void motor_stop(void) { motor_control(0, 0); }

## 四、部署配置

### 1. Docker Compose docker-compose.yml

YAML version: '3.8' services: # MQTT Broker emqx: image: emqx/emqx:5.7.0 container_name: clawlink-emqx restart: always environment: - TZ=Asia/Shanghai - EMQX_DASHBOARD_DEFAULT_USER_PASSWORD=${MQTT_ADMIN_PASS} ports: - "${MQTT_PORT}:1883" - "${MQTT_WEB_PORT}:18083" volumes: - ./data/emqx/data:/opt/emqx/data - ./data/emqx/log:/opt/emqx/log networks: - clawlink-net # 本地LLM服务 ollama: image: ollama/ollama:latest container_name: clawlink-ollama restart: always environment: - TZ=Asia/Shanghai - OLLAMA_HOST=0.0.0.0 ports: - "11434:11434" volumes: - ./data/ollama/models:/root/.ollama/models networks: - clawlink-net privileged: true # 脊椎层后端服务 backend: build: ./raspberrypi-spine/backend container_name: clawlink-backend restart: always environment: - TZ=Asia/Shanghai - MQTT_BROKER=emqx - MQTT_PORT=${MQTT_PORT} - MQTT_EDGE_USER=${MQTT_EDGE_USER} - MQTT_EDGE_PASS=${MQTT_EDGE_PASS} - OLLAMA_BASE_URL=http://ollama:11434/api - OLLAMA_DEFAULT_MODEL=${OLLAMA_DEFAULT_MODEL} - CAMERA_INDEX=${CAMERA_INDEX} - RPI_HOST_IP=${RPI_HOST_IP} ports: - "8001:8001" volumes: - ./raspberrypi-spine/backend/src:/app devices: - /dev/video0:/dev/video0 - /dev/gpiomem:/dev/gpiomem privileged: true depends_on: - emqx - ollama networks: - clawlink-net # 脊椎层前端服务 frontend: image: nginx:alpine container_name: clawlink-frontend restart: always ports: - "8000:80" volumes: - ./raspberrypi-spine/frontend:/usr/share/nginx/html depends_on: - backend networks: - clawlink-net networks: clawlink-net: driver: bridge

### 2. 环境变量模板 .env.example

TOML # 基础配置 RPI_HOST_IP=192.168.3.100 TZ=Asia/Shanghai # MQTT配置 MQTT_ADMIN_PASS=ClawLink@2026 MQTT_EDGE_USER=clawlink-edge MQTT_EDGE_PASS=ClawLinkEdge@2026 MQTT_DEVICE_PASS=ClawLinkDevice@2026 MQTT_PORT=1883 MQTT_WEB_PORT=18083 # LLM配置 OLLAMA_DEFAULT_MODEL=llama3:8b-instruct-q4_0 # 服务配置 CAMERA_INDEX=0 WEB_PORT=8000 API_PORT=8001

## 五、快速部署指南

### 1. 树莓派环境准备

Bash # 更新系统 sudo apt update && sudo apt upgrade -y # 安装Docker curl -fsSL https://get.docker.com ¦ sh sudo usermod -aG docker $USER newgrp docker # 克隆项目 git clone https://github.com/你的用户名/ClawLink-TriLayer.git cd ClawLink-TriLayer # 配置环境变量 cp .env.example .env # 编辑.env文件，修改RPI_HOST_IP为树莓派实际IP nano .env # 启动服务 docker compose up -d # 下载LLM模型 docker exec -it clawlink-ollama ollama pull llama3:8b-instruct-q4_0

### 2. ESP32固件烧录

Bash # 进入ESP32代码目录 cd esp32-periphery # 设置目标芯片 idf.py set-target esp32s3 # 编译固件 idf.py build # 烧录固件（替换串口为实际设备） idf.py -p /dev/ttyUSB0 flash monitor

### 3. OpenClaw插件安装

Bash # 进入大脑层目录 cd openclaw-brain # 执行安装脚本 chmod +x install_skill.sh ./install_skill.sh # 配置OpenClaw nano ~/.openclaw/config.toml # 添加以下配置： [clawlink_trilayer] mqtt_broker = "你的树莓派IP" mqtt_port = 1883 mqtt_user = "clawlink-upper" mqtt_pass = "ClawLinkUpper@2026" raspberrypi_api = "http://你的树莓派IP:8001"

### 4. 验证部署

访问Web前端：http://树莓派IP:8000

访问MQTT后台：http://树莓派IP:18083（账号admin，密码为.env中的MQTT_ADMIN_PASS）

测试指令：在OpenClaw中发送「采集所有设备温湿度并汇总」

## 六、二次开发指南

### 新增传感器

在ESP32末梢层的sensors.c中添加传感器驱动函数

更新sensor_data_t结构体，添加新传感器字段

在sensor_collect_task中读取新传感器数据

在MQTT上报逻辑中包含新传感器数据

### 新增执行器

在ESP32末梢层的actuators.c中添加执行器控制函数

在handle_mqtt_command中添加新指令的解析逻辑

在树莓派脊椎层的mqtt_client.py中添加新指令的转发逻辑

在Web前端中添加新执行器的控制界面

### 新增LLM能力

在树莓派脊椎层的llm_agent.py中添加新的推理函数

在FastAPI中新增接口暴露该能力

在OpenClaw插件中添加新指令的映射逻辑

## 七、开源协议

本项目采用 MIT License 开源，你可以自由使用、修改、分发本项目的代码，包括商业用途，仅需保留原作者的版权声明。

## 八、致谢

ESP-IDF：ESP32硬件驱动基础

OpenClaw：AI任务调度框架

YOLOv8：计算机视觉推理

Ollama：本地大模型推理

FastAPI：后端API开发

Vue3：前端界面开发

EMQX：MQTT消息中间件

|（注：文档部分内容可能由 AI 生成)